pH Determination

Title

Determination of pH in Fresh Wastewater and Saturated Wastewater

Aim of the Experiment

To determine and compare the pH values of fresh wastewater and saturated wastewater samples.

Introduction

pH is an important parameter in wastewater quality analysis because it indicates the acidity or alkalinity of water. The pH scale ranges from 0 to 14:

Monitoring pH in wastewater is essential because extreme acidic or alkaline conditions can affect biological treatment processes, damage equipment, and harm aquatic organisms when wastewater is discharged into the environment.

The acceptable pH range for most wastewater treatment systems and environmental discharge standards is usually between 6.5 and 8.5.

Apparatus and Materials

- pH meter

- Beakers

- Fresh wastewater sample

- Saturated wastewater sample

- Distilled water

- Tissue paper

Procedure

- The pH meter was calibrated using standard buffer solutions.

- Fresh wastewater sample was poured into a clean beaker.

- The electrode of the pH meter was immersed into the sample.

- The pH reading was allowed to stabilize and then recorded.

- The electrode was rinsed with distilled water and dried using tissue paper.

- The same procedure was repeated for the saturated wastewater sample.

- All observations and readings were recorded carefully.

Results

Sample Type

Fresh Wastewater

pH value 7.44

Saturated Wastewater

pH value 7.4

Discussion

The pH value of the fresh wastewater was found to be 7.44, while the saturated wastewater had a pH value of 7.42. Both samples were slightly alkaline because their values were slightly above neutral pH 7.

The small difference between the two samples indicates that saturation did not significantly affect the acidity or alkalinity of the wastewater. The pH values fall within the acceptable range for wastewater discharge and biological treatment processes.

A stable pH near neutrality is favourable because:

It supports microbial activity during biological treatment. It reduces corrosion of pipes and equipment. It minimizes harmful effects on aquatic life after discharge.

The results therefore suggest that the wastewater samples were chemically stable with respect to acidity and alkalinity.

Conclusion

The experiment successfully determined the pH of both wastewater samples. The fresh wastewater had a pH of 7.44 and the saturated wastewater had a pH of 7.42. Both samples were slightly alkaline and within the recommended range for wastewater treatment and environmental safety.

Precautions

- The pH meter was calibrated before use.

- The electrode was rinsed with distilled water after every reading to avoid contamination.

- Clean beakers were used for each sample.

- Readings were recorded only after stabilization of the pH meter.

Title

Determination of Electrical Conductivity (EC) in Fresh Wastewater and Saturated Wastewater

Aim of the Experiment

To determine and compare the Electrical Conductivity (EC) of fresh wastewater and saturated wastewater samples.

Introduction

Electrical Conductivity (EC) is the measure of the ability of water to conduct electric current. It depends on the presence of dissolved ions such as salts, minerals, acids,  and alkalis in water. EC is an important parameter in water quality assessment because it indicates the concentration of ionic substances in wastewater.

EC is commonly expressed in microsiemens per centimeter (μS/cm). High EC values indicate high concentrations of dissolved ions, which may originate from industrial waste, sewage, fertilizers, or chemical pollutants.

Monitoring EC is important in wastewater treatment, environmental management, irrigation, and industrial applications.

Apparatus and Materials

- EC meter (Conductivity meter)

- Beakers

- Distilled water

- Fresh wastewater sample

- Saturated wastewater sample

- Tissue paper

- Stirring rod

Procedure

- Fresh wastewater and saturated wastewater samples were collected in clean beakers.

- The EC meter was calibrated using standard solution according to laboratory instructions.

- The probe was rinsed with distilled water and wiped dry.

- The probe was immersed into the fresh wastewater sample and the EC reading was recorded

- after stabilization.

- The procedure was repeated for the saturated wastewater sample.

- All results were recorded in μS/cm.

Results

Sample Type

Fresh Wastewater

EC Value (μS/cm) 2420

Saturated Wastewater

EC Value (μS/cm) 3970

Discussion

- The results show that the saturated wastewater had a higher Electrical Conductivity value (3970μS/cm) compared to the fresh wastewater (2420 μS/cm). This indicates that the saturated wastewater contained a greater amount of dissolved ionic substances. The high EC values may be due to the presence of: Dissolved salts, Industrial chemicals, Domestic sewage, Organic and inorganic pollutants

- The fresh wastewater sample had a lower EC value, suggesting a lower concentration of dissolved ions. However, both samples recorded relatively high EC values, indicating significant levels of dissolved contaminants.

- High EC in wastewater can:

Affect aquatic ecosystems, Reduce soil quality during irrigation, Cause corrosion and scaling in pipes, Indicate pollution or poor water quality, The increase in EC from fresh wastewater to saturated wastewater suggests accumulation of dissolved substances in the saturated sample.

Conclusion

The experiment successfully determined the Electrical Conductivity of the wastewater samples. The saturated wastewater recorded the highest EC value of 3970 μS/cm, while the fresh wastewater recorded 2420 μS/cm. The results indicate that saturated wastewater contains a higher concentration of dissolved ionic materials. Proper treatment is therefore necessary before wastewater discharge or reuse.

LABORATORY REPORT ON TOTAL DISSOLVED SOLIDS (TDS) ANALYSIS OF

WASTEWATER

Title

Determination of Total Dissolved Solids (TDS) in Fresh Wastewater and Saturated Wastewater

Aim of the Experiment

To determine and compare the Total Dissolved Solids (TDS) concentration in fresh wastewater and saturated wastewater samples.

Introduction

Total Dissolved Solids (TDS) refer to the total amount of dissolved inorganic and organic substances present in water. These substances include minerals, salts, metals, cations, and anions dissolved in water. TDS is an important parameter used in assessing water quality because high concentrations may affect aquatic life, industrial processes, irrigation systems, and human health. TDS values are usually expressed in milligrams per liter (mg/L). Wastewater with high TDS levels

may indicate contamination from domestic, agricultural, or industrial activities.

Apparatus and Materials

- TDS meter

- Beakers

- Distilled water

- Fresh wastewater sample

- Saturated wastewater sample

- Stirring rod

- Tissue paper

Procedure

- Two wastewater samples were collected: Fresh wastewater, Saturated wastewater

- The TDS meter was calibrated using distilled water.

- The electrode of the TDS meter was rinsed with distilled water and wiped clean.

- The electrode was immersed into the fresh wastewater sample and the reading was recorded after stabilization.

- The procedure was repeated for the saturated wastewater sample.

- All readings were recorded in mg/L.

Results

Sample Type

Fresh Wastewater

TDS Value (mg/L) 1200

Saturated Wastewater

TDS Value (mg/L) 1990

LABORATORY REPORT ON PHOSPHATE ANALYSIS OF WASTEWATER

Title

Determination of Phosphate Concentration in Fresh Wastewater and Saturated Wastewater

Aim of the Experiment

To determine and compare the phosphate concentration in fresh wastewater and saturated wastewater samples.

Introduction

Phosphate is an essential nutrient commonly found in wastewater due to domestic sewage, detergents, fertilizers, industrial effluents, and decomposition of organic matter. In water quality analysis, phosphate is an important parameter because excessive phosphate levels can lead to environmental problems such as eutrophication.

Eutrophication is the excessive growth of algae and aquatic plants caused by nutrient enrichment in water bodies. This process reduces dissolved oxygen levels and negatively affects aquatic organisms.

Phosphate concentration is commonly expressed in milligrams per liter (mg/L).

Apparatus and Materials

- Spectrophotometer or colorimeter

- Test tubes

- Beakers

- Pipettes

- Distilled water

- Fresh wastewater sample

- Saturated wastewater sample

- Phosphate reagent

- Measuring cylinder

Procedure

- Wastewater samples were collected in clean containers.

- Appropriate volumes of fresh wastewater and saturated wastewater samples were measured into separate test tubes.

- Phosphate reagent was added to each sample according to laboratory procedures.

- The samples were mixed thoroughly and allowed to react for the required time for colour development.

- The absorbance of each sample was measured using a spectrophotometer/colorimeter.

- Phosphate concentrations were determined and recorded in mg/L.

Results

Sample Type

Phosphate Concentration (mg/L)

Fresh Wastewater

1.75

Saturated Wastewater

3.9

Discussion

- The results show that the saturated wastewater had a higher phosphate concentration (3.9 mg/L) compared to the fresh wastewater (1.75 mg/L). This indicates that the saturated wastewater contained more nutrient pollutants and dissolved phosphate compounds.

- The elevated phosphate levels may originate from: Detergents and soaps, Domestic sewage, Decomposition of organic matter The fresh wastewater sample showed a lower phosphate concentration, suggesting fewer dissolved nutrients compared to the saturated wastewater. High phosphate concentrations in wastewater can cause: Eutrophication in rivers and lakes Excessive algal growth Depletion of dissolved oxygen Death of aquatic organisms Deterioration of water quality The increase in phosphate concentration in saturated wastewater suggests accumulation of nutrient-rich substances in the water.

Conclusion

The experiment successfully determined the phosphate concentration in the wastewater samples. Saturated wastewater recorded the highest phosphate concentration of 3.9 mg/L, while fresh wastewater recorded 1.75 mg/L. The results  indicate that saturated wastewater contains a greater amount of nutrient pollutants. Proper wastewater treatment is therefore necessary before discharge into the environment.

Precautions

- Clean apparatus were used to avoid contamination.

- Accurate measurements of reagents and samples were maintained.

- Samples were properly mixed before analysis.

- The spectrophotometer/colorimeter was calibrated before use.

- Readings were taken carefully after proper color development.

References

- APHA (American Public Health Association). Standard Methods for the Examination of Water and Wastewater.

- Metcalf & Eddy. Wastewater Engineering: Treatment and Reuse.

- World Health Organization (WHO) Water Quality Guidelines